Audio Preprocessing Steps & Their Importance

1. Remove Ellie (Interviewer Speech)

What it does

Removes the interviewer’s voice using transcript timestamps.

Why it is important

Your study analyzes participant behaviour. If Ellie remains in the audio:

- speaking rate becomes incorrect

- pause detection becomes incorrect

- pitch statistics get mixed

- energy statistics become meaningless

Example:

Participant: I don't know

Ellie: Why do you think that?

Participant: I just feel tired

If Ellie stays, the system thinks two speakers belong to the same person.

Research justification

Ensures extracted features represent only the participant’s behavioural speech patterns.

2. Extract Participant Segments

What it does

Cuts audio segments where the participant is speaking.

audio[start_time : stop_time]

Why it is important

The transcript provides accurate alignment between speech and time.

Using timestamps allows:

- precise speech extraction

- no need for speaker diarization

- consistent segmentation across dataset

Research justification

Ensures high-quality segmentation using ground-truth timestamps.

3. Concatenate Participant Segments

What it does

Merges all participant speech segments into one continuous audio signal.

Why it is important

This reconstructs the participant's entire speech timeline.

It preserves:

- natural speaking rhythm

- pause structure

- conversational behaviour

If segments remain separate, later analysis becomes difficult.

Research justification

Allows extraction of global behavioural speech features across the entire interview.

4. Convert to Mono

What it does

Converts stereo audio into a single channel.

Why it is important

Speech features are calculated from one waveform.

Stereo recordings can cause:

- duplicate signals

- channel imbalance

- incorrect energy measurements

Most speech analysis models assume mono audio.

Research justification

Ensures consistent channel representation for acoustic feature extraction.

5. Resample to 16 kHz

What it does

Converts all recordings to 16,000 samples per second.

Why it is important

Speech information mostly lies within:

0 Hz – 8000 Hz

16 kHz sampling captures this entire range.

Benefits:

- consistent feature extraction

- lower computational cost

- standard for speech processing

Most speech datasets and models use 16kHz.

Research justification

Ensures uniform temporal resolution across recordings.

6. Amplitude Normalization

What it does

Scales audio so the maximum amplitude is consistent.

Example:

audio = audio / max(abs(audio))

Why it is important

Recording loudness varies across participants.

Without normalization:

- energy features become biased

- louder recordings appear more "active"

- models learn recording conditions instead of behaviour

Normalization removes recording-level bias.

Research justification

Ensures comparability of acoustic energy features across participants.

7. Mild Noise Reduction (Optional)

What it does

Reduces background noise.

Examples:

- microphone noise

- environmental noise

Why it is important

Noise can distort:

- MFCC

- spectral features

- pitch detection

However excessive filtering can damage:

- jitter

- shimmer

- natural speech variation

So only light denoising is recommended.

Research justification

Improves signal quality while preserving natural speech characteristics.

Save Cleaned Audio

What it does

Stores processed audio files for later stages.

Example:

clean_audio/

300.wav

301.wav

Why it is important

Preprocessing should only happen once.

Later steps will use these files for:

- phase segmentation

- feature extraction

- model training

Research justification

Ensures reproducibility and consistent data usage across experiments.

Summary Table

| Step | Purpose |
| --- | --- |
| Remove Ellie | isolate participant behaviour |
| Extract segments | accurate speech extraction |
| Concatenate segments | reconstruct full speech timeline |
| Convert to mono | consistent channel representation |
| Resample 16kHz | standard speech processing rate |
| Normalize amplitude | remove loudness bias |
| Noise reduction | improve signal quality |
| Save cleaned audio | reproducible dataset |